BASIC WORKFLOW FOR PROCESSING EPHYS DATA:

- Acquire data

- In Matlab (*), run_kilosort.m (should be flexible to run across all experiments)

*NOTE: See below on setting this up: Requires GPU and later versions of Matlab (>2016, I think).

- Batch_run_kilosort.m is specifically designed to batch-run kilosort and uses experiment-specific setup will go through all

- Run Phy for spike sorting (see notes below)

- Move the data directory onto server

- Further data extraction with my own analysis tools

  - batch_extractEphys.m

CURRENT HARDWARE SETUP:

- Sync pulses for behavior from statescript at each centerIN

- TTL split to openEphys (port x) and TDT

- Camera Sync pulses sent from Basler to openEphys (port x)

- Laser diode (632nm) TTL sync sent to openEphys (port x)

ACQUISITION NOTES:

- Some comments on order during running

- Orientation convention has been intan chip on animal’s right

- Fiber has been oriented to left during surgery using David arrays

- Naming Convention

  - AnimalName _autoDateFill_taskName (openEphys will dump files into folder with this structure)

ANALYSIS PIPELINE:

Spike Sorting:

- NOTE: I’m currently using Kilosort 1.0 (https://github.com/cortex-lab/KiloSort). I have not tried Kilosort 2.0 or other spike sorting software (i.e. mountainSort). This requires a GPU and setting up compatibility between CUDA and Matlab. Try to follow instructions. Should eventually move to KiloSort 2.

SETTING UP KILOSORT: (should only need to be done once for each type of array used)

- First, download Kilosort (https://github.com/cortex-lab/KiloSort) and follow instructions.

- createChannelMapFile.m has to be made once for each different type of array to create the geometric configuration used for spike sorting. I made one for the 32-channel arrays from Innovative Neurophysiology. [PIC HERE and EXAMPLE HERE]. Follow their example scripts to customize your own.

- Changes I had to make to their functions to make things work (My guess is that openEphys must have changed their naming convention after these functions were written):

- In convertOpenEphysToRawBinary.m (line 12):  fs{j} = dir(fullfile(ops.root, sprintf('*CH%d.continuous', j) )); % removed an underscore here and *

RUNNING KILSORT: (this creates a bunch of .npy files that are used for manual spike sorting)

- Make sure to populate spreadsheet and save .csv to computer

- run_kilosort.m will batch process all files listed in the .csv:

  - Make sure the local environment is setup properly (open run_kilosort.m and fill in appropriately)

  - Edit the StandardConfig.m file which will have kilosort options (I haven’t explored most of these, but definitely change channel count, sampling rate stuff to match your hardware).

  - getExpData.m is just a function to extract data from your .csv files. Needs to be set up to match your particular needs

  - Double check to make sure the string being created in the ops.root variable matches the directory for each experiment.

- This produces a bunch of .py files tha twill later be used for manual spike sorting.

MANUAL SPIKE SORTING: (uses phy: Requires MATLAB 2016 or GREATER)

- Setting up the environment for spike sorting: https://github.com/cortex-lab/phy

- First, check out Instructions for use: https://phy.readthedocs.io/en/latest/

- Open Anaconda Prompt terminal window

- Type ‘activate phy’

- Change directories to where the params.py file exists for this particular recording session (i.e. cd \testData\CD190819A_190917_01). NOTE: if changing directories to an external drive, first type name of drive (i.e. ‘F:’). Then cd into the directory.

- Type ‘phy template-gui params.py’

- Start sorting: follow guidelines in gui/https://github.com/kwikteam/phy-contrib/blob/master/docs/template-gui.md

- Saving will create a rez.mat file

EPHYS DATA STRUCTURES:

Batch_extractEphys.m is the main function that will extract all relevant data from the openEphys experiments and runs the following routines:

Opts.rootDir = root Directory where all the experimental data is

Opts.samplingRate = sampling rate in KHz (30000)

Opts.acc.res = resolution to downsample accelerometer (in ms)

Opts.sync.optoChan = BNC port # where laser TTL is set (1)

Opts.sync.syncChan = BNC port # where stateScript Sync is set (2)

Opts.sync.camChan = BNC port # where Basler TTL frame sync is sent (3)

Opts.optoID.window = spikes saved in this window relative to optoID pulse (ms) [-200 500]

Eventually have something set default opts (load_default_opts)

createSpkFile(fName,opts) will create and save a SPK for an experiment.

SPK.fileName = animalName_date_session# identifier

SPK.opts = Contains all options for ephys Extraction

SPK.neuron{n#}.spikeTimes = Timing for each neuron (openEphys Clock)

.template (time X channelID) waveform template (time [get units] x channels)

.electrode = Electrode number that had the maximum deflection (Could be used later for choosing only best neuron per electrode). Eventually integrate map somewhere

SPK.mua{n#}. Same as above but for signal designated as multiunit activity

createAccFile(fName,opts) (Accelerometer data)

createSyncFile(fName,opts) creates SYN (sync) data structure)

SYN.fileName = = animalName_date_session# identifier

SYN.opts = Contains all options for ephys Extraction

SYN.syncTimes = sync times for behavior (openEphys Clock (s))

SYN.camTimes = camera sync times (openEphys Clock (s))

SYN.optoTimes = optoID times (openEphys Clock (s))

SYN.lasDur = duration of laser for optoID (in ms)

- LFP (TBD)

- WAVE (TBD)

Analysis code:

get_optoID(SPK,SYN,opts)

SPK and SYN give spike and sync structures. opts give parameters for plotting. Produces a .ps plot for each session with laser ID raster, sdf, stats, electrode ID, waveforms. Returns vector of neuron# that are ID’d (0’s and 1’s) based on statistical parameters in opts.

Opts.plot_window = [min max]: min and max of plotting window (ms)

Opts.plot = 0 or 1: To make a new .ps plot or not

Opts.anWin = [min max]: analysis window for stats (ms) (paired t-test, one tailed)

Opts.baseWin = [min max]: baseline window for stats (ms)

Opts.gFilter = standard deviation of gaussian for convolution (1 std, in ms)

TBD: Should compare waveforms for optoID’d units (Look at Jerimiah Cohen and Uchida’s paper for their methods. Also look at Ally’s paper for optoID stat criteria in striatum.

PS PLOT SCRIPTS: (in ps_plots directory)

batch_optoID_ps.m

- This batch processes all ephys data in .csv file and creates optoID .ps plots

Code for working with Kilosort and getting waveforms, etc:

https://github.com/cortex-lab/spikes